Configure OSPF MD5 authentication and support SSH connections

2. Configure Router

Router 1

Router>en

Router#conf t

Router(config)#host r1

r1(config)#int g0/1

r1(config-if)# ip address 192.168.1.1 255.255.255.0

r1(config-if)#no shut

r1(config-if)#int s0/0/0

r1(config-if)#ip address 10.1.1.1 255.255.255.252

r1(config-if)#no shut

Router 2

Router>en

Router#conf t

Router(config)#host r2

r2(config)#int s0/0/0

r2(config-if)#ip address 10.1.1.2 255.255.255.252

r2(config-if)#no shut

r2(config-if)#int s0/0/1

r2(config-if)#ip address 10.2.2.2 255.255.255.252

r2(config-if)#no shut

Router 3

Router>en

Router#conf t

Router(config)#host r3

r3(config)#int g0/1

r3(config-if)#ip address 192.168.3.1 255.255.255.0

r3(config-if)#no shut

r3(config-if)#int s0/0/1

r3(config-if)#ip address 10.2.2.1 255.255.255.252

r3(config-if)#no shut

3. Configure OSPF

Router 1

r1(config)#router ospf 1

r1(config-router)#network 192.168.1.0 0.0.0.255 area 0

r1(config-router)#network 10.1.1.0 0.0.0.3 area 0

r1(config-router)#exit

r1(config)#exit

r1#sh ip route

Router 2

r2(config)#router ospf 1

r2(config-router)#network 10.1.1.0 0.0.0.3 area 0

r2(config-router)#network 10.2.2.0 0.0.0.3 area 0

r2(config-router)#exit

r2(config)#exit

r2#sh ip route

Router 3

r3(config)#router ospf 1

r3(config-router)#network 192.168.3.0 0.0.0.255 area 0

r3(config-router)#network 10.2.2.0 0.0.0.3 area 0

r3(config-router)#exit

r3(config)#exit

r3#sh ip route

4. Configure MD5 Authentication

Router 1

r1(config)#router ospf 1

r1(config-router)#int s0/0/0

r1(config-router)#ip ospf message-digest-key 1 md5 ty33

r1(config-router)#exit

r1(config)#do sh ip ospf int

Router 2

r2(config)#router ospf 1

r2(config-router)#int s0/0/0

r2(config-router)#ip ospf message-digest-key 1 md5 ty33

r2(config-router)#int s0/0/1

r2(config-router)#ip ospf message-digest-key 1 md5 ty33

r2(config-router)#exit

r2(config)#do sh ip ospf int

r3(config)#router ospf 1

r3(config-router)#int s0/0/1

r3(config-router)#ip ospf message-digest-key 1 md5 ty33

r3(config-router)#exit

r3(config)#do sh ip ospf int

5. Configure Router 3 to support SSH connections

Router 3

r3(config)#ip domain-name tyitsecurity.com

r3(config)#enable secret ty33

Exit the connections and enable the router again. It will ask for password, enter ty33, it will not be visible but the system will accept it. Once you press enter, it will take you to config mode of the router.

r3(config)#username admin123 privilege 15 secret ty33

r3(config)#line vty 0 4

r3(config-line)#login local

r3(config-line)#transport input ssh

r3(config-line)#exit

r3(config)#crypto key generate rsa

How many bits in the modulus? 1024

r3(config)#do sh ip ssh

Check results

Open Command Prompt of PC – C.

Enter telnet 192.168.3.1, the connection will open. Enter ssh -l admin123 192.168.3.1

Password will be asked type ty23 which will not be visible but will be accepted by the system. Router 3 configuration is now accessible from PC – C

Configure AAA authentication on routers.

2. Configure Router

Router 1

Router>en

Router#conf t

Router(config)#host r1

r1(config)#int g0/1

r1(config-if)# ip address 192.168.1.1 255.255.255.0

r1(config-if)#no shut

r1(config-if)#int s0/0/0

r1(config-if)#ip address 10.1.1.2 255.255.255.252

r1(config-if)#no shut

Router 2

Router>en

Router#conf t

Router(config)#host r2

r2(config)#int g0/0

r2(config-if)#ip address 192.168.2.1 255.255.255.0

r2(config-if)#no shut

r2(config-if)#int s0/0/0

r2(config-if)#ip address 10.1.1.1 255.255.255.252

r2(config-if)#no shut

r2(config-if)#int s0/0/1

r2(config-if)#ip address 10.2.2.1 255.255.255.252

r2(config-if)#no shut

Router 3

Router>en

Router#conf t

Router(config)#host r3

r3(config)#int g0/1

r3(config-if)#ip address 192.168.3.1 255.255.255.0

r3(config-if)#no shut

r3(config-if)#int s0/0/1

r3(config-if)#ip address 10.2.2.2 255.255.255.252

r3(config-if)#no shut

3. Configure RIP on routers

Router 1

r1(config)#router rip

r1(config-router)#network 192.168.1.0

r1(config-router)#network 10.1.1.0

r1(config-router)#exit

r1>sh ip route

Router 2

r2(config)#router rip

r2(config-router)#network 10.1.1.0

r2(config-router)#network 192.168.2.0

r2(config-router)#network 10.2.2.0

r2(config-router)#exit

r2>sh ip route

Router 3

r3(config)#router rip

r3(config-router)#network 192.168.3.0

r3(config-router)#network 10.2.2.0

r3(config-router)#exit

r3>sh ip route

4. Configure local AAA authentication on Router 1

Router 1

r1>en

r1#conf t

r1(config)#username admin123 secret ty33

r1(config)#aaa new-model

r1(config)#aaa authentication login default local

r1(config)#line console 0

r1(config-line)#login authentication default

Username is admin123 and password is ty33.

5. Configure local AAA authentication for vty lines on Router 1

From security.com it is

Router 1

r1>en

r1#conf t

r1(config)#ip domain-name security.com

r1(config)#crypto key generate rsa

How many bits in the modulus will be asked, enter 1024.

r1(config)#aaa authentication login SSH-LOGIN local

r1(config)#line vty 0 4

r1(config-line)#login authentication SSH-LOGIN

r1(config-line)#transport input ssh

r1(config-line)#exit

Go to PC – A Command Prompt and enter ssh -l admin123 192.168.1.1. It will ask for password, enter ty33.

6. Configure service-based AAA authentication using TACACS+ on Router 2

Go to services of TACACS+ Server and then select AAA. Switch on the service and enter the client IP and password. Setup username and password as well.

Router 2

r2>en

r2#conf t

r2(config)#username admin secret admin123

r2(config)#tacacs-server host 192.168.2.2

r2(config)#tacacs-server key pwdr2

r2(config)#aaa new-model

r2(config)#aaa authentication login default group tacacs+ local

r2(config)#line console 0

r2(config-line)#login authentication default

User Access Verification is asked enter the credentials of the TACACS+ Server.

Username is admin and password is admin123 which is also the username and password set for TACACS Server.

7. Configure server-based AAA authentication using RADIUS on Router 3

Go to services of RADIUS Server and then select AAA. Switch on the service and enter the client IP and

password. Setup username and password as well.

Router 3

r3>en

r3#conf t

r3(config)#username admin1 secret admin321

r3(config)#radius-server host 192.168.3.2

r3(config)#radius-server key pwdr3

r3(config)#aaa new-model

r3(config)#aaa authentication login default group radius local

r3(config)#line console 0

r3(config-line)#login authentication default

User Access Verification is asked enter the credentials of the RADIUS Server.

Username is admin1 and password is admin321 which is also set in radius server.

Configure a Zone-Based Policy Firewall (ZPF)

B) Configure Router

Router 1

Router>en

Router#conf t

Router(config)#host r1

r1(config)#int g0/1

r1(config-if)# ip address 192.168.1.1 255.255.255.0

r1(config-if)#no shut

r1(config-if)#int s0/0/0

r1(config-if)#ip address 10.1.1.1 255.255.255.252

r1(config-if)#no shut

Router 2

Router>en

Router#conf t

Router(config)#host r2

r2(config)#int s0/0/0

r2(config-if)#ip address 10.1.1.2 255.255.255.252

r2(config-if)#no shut

r2(config-if)#int s0/0/1

r2(config-if)#ip address 10.2.2.2 255.255.255.252

r2(config-if)#no shut

Router 3

Router>en

Router#conf t

Router(config)#host r3

r3(config)#int g0/1

r3(config-if)#ip address 192.168.3.1 255.255.255.0

r3(config-if)#no shut

r3(config-if)#int s0/0/1

r3(config-if)#ip address 10.2.2.1 255.255.255.252

r3(config-if)#no shut

C) Configure Static Routing

Go to Config of Router 1 → Static under Routing and add

the details that are

Network: 0.0.0.0

Mask: 0.0.0.0

Next Hop: 10.1.1.2

r1#show ip route

Router 2

Network: 192.168. 1.0

Mask: 255.255.255.0

Next Hop: 10.1.1.1

Network: 192.168.3.0

Mask: 255.255.255.0

Next Hop: 10.2.2.1

r2#show ip route

Router 3

Network: 0.0.0.0

Mask: 0.0.0.0

Next Hop: 10.2.2.2

r3#show ip route

D) Password Authentication of Router

Router 1

r1(config)#enable secret ty23

r1(config)#line console 0

r1(config-line)#password ty23

r1(config-line)#login

r1(config-line)#exit

Exit from the connection. Under User Access Verification password is asked, enter ty23, it will not be seen but the system will accept it. Connection to the router will open again.

Router 1

r1#conf t

r1(config)#username admin1 secret admin123

r1(config)#aaa new-model

r1(config)#aaa authentication login default local

r1(config)#line console 0

r1(config-line)#login authentication default

r1(config-line)#end

End the connection completely. Again, User Access Verification will come, enter Username as admin1and Password as admin123. After enabling the router if again a password is asked enter ty23.

Router 1

r1(config)#ip domain-name security.com

r1(config)#crypto key generate rsa

How many bits in the modulus will be asked, enter

1024.

r1(config)#aaa authentication login sshlogin local

r1(config)#line vty 0 4

r1(config-line)#login authentication sshlogin

r1(config-line)#transport input ssh

r1(config-line)#end

E) Enable security technology package on Router3

Router 3

r3(config)#license boot module c1900 technology-package securityk9

Accept the agreement by typing yes.

r3#reload

Confirm the reload.

r3#show version

F) Create firewall zone on Router3

r3(config)#zone security IN-ZONE

r3(config-sec-zone)#exit

r3(config)#zone security OUT-ZONE

r3(config-sec-zone)#exit

G) Identify traffic using a class-map

r3(config)# access-list 101 permit ip

192.168.3.0 0.0.0.255 any

r3(config)# class-map type inspect match-all IN-NET-CLASS-MAP

r3(config-cmap)# match access-group 101

r3(config-cmap)# exit

H) Firewall policies

r3(config)# policy-map type inspect IN-2-OUT-PMAP

r3(config-pmap)# class type inspect IN-NET-CLASS-MAP

r3(config-pmap-c)# inspect

r3(config-pmap-c)# exit

r3(config-pmap)# exit

r3(config)# zone-pair security IN-2-OUT-ZPAIR source IN-ZONE destination OUTZONE

r3(config-sec-zone-pair)# service-policy type inspect IN-2-OUT-PMAP

r3(config-sec-zone-pair)# exit

r3(config)# interface g0/1

r3(config-if)# zone-member security IN-ZONE

r3(config-if)# exit

r3(config)# interface s0/0/1

r3(config-if)# zone-member security OUT-ZONE

r3(config-if)# exit

I) Test firewall functionality from IN-ZONE to OUT- ZONE

Open Command Prompt of PC – C and ping 192.168.1.3 that is PC – A (Server). It should be successful.

In CLI mode of Router 3 type,

r3#show policy-map type inspect zone-pair sessions The details regarding the policy should be displayed.

Go to Web Browser of PC - C enter

http://192.168.1.3 and click on Go, Cisco Packet Tracer page will be displayed.

- Test firewall functionality from OUT-ZONE to IN-ZONE

Open Command Prompt of PC – A and ping 192.168.3.3 that is PC – C. It should fail.